| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 23rd Meeting, by teleconference, 7–16 July 2021 | Document: JVET-W0183-v1 |
| --- | --- |

| Title: | Maximum picture rates for level 6.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | frank@bossentech.com |
| Source: | Editor |  |  |

_____________________________

# Abstract

Section A.4.3 of the VVC specification is informative and provides examples of the relationship between level, picture size, and maximum picture rate. Values for level 6.3 are provided in this document to support the disposition of a related ballot comment.

# Introduction

Maximum pictures rates for level 6.3 are provided in the tables below. Moving values for levels 6.0, 6.1, and 6.2 into these new tables may be considered.

Table X – Maximum picture rates (pictures per second) at the Main tier, level 6.3 for some example picture sizes when MinCbSizeY is equal to 64

| Level: |  |  |  | 6.3 |
| --- | --- | --- | --- | --- |
| Max luma picture size (samples): |  |  |  | 80 216 064 |
| Max luma sample rate (samples/sec) |  |  |  | 4 812 963 840 |
| Format nickname | Luma width | Luma height | Luma picture size |  |
| 1080 HD | 1 920 | 1 080 | 2 088 960 | 300.0 |
| 2K×1K | 2 048 | 1 024 | 2 097 152 | 300.0 |
| 2K×1080 | 2 048 | 1 080 | 2 228 224 | 300.0 |
| 4XGA | 2 048 | 1 536 | 3 145 728 | 300.0 |
| 16VGA | 2 560 | 1 920 | 4 915 200 | 300.0 |
| 3616×1536 (2.35:1) | 3 616 | 1 536 | 5 603 328 | 300.0 |
| 3672×1536 (2.39:1) | 3 680 | 1 536 | 5 701 632 | 300.0 |
| 3840×2160 (4*HD) | 3 840 | 2 160 | 8 355 840 | 300.0 |
| 4K×2K | 4 096 | 2 048 | 8 388 608 | 300.0 |
| 4096×2160 | 4 096 | 2 160 | 8 912 896 | 300.0 |
| 4096×2304 (16:9) | 4 096 | 2 304 | 9 437 184 | 300.0 |
| 4096×3072 | 4 096 | 3 072 | 12 582 912 | 300.0 |
| 7680×4320 | 7 680 | 4 320 | 33 423 360 | 144.0 |
| 8192×4096 | 8 192 | 4 096 | 33 554 432 | 143.4 |
| 8192×4320 | 8 192 | 4 320 | 35 651 584 | 135.0 |
| 11520×6480 | 11 520 | 6 480 | 74 649 600 | 64.0 |
| 12288×6144 | 12 288 | 6 144 | 75 497 472 | 63.7 |
| 12288×6480 | 12 288 | 6 480 | 79 626 240 | 60.0 |

Table X – Maximum picture rates (pictures per second) at the High tier, level 6.3 for some example picture sizes when MinCbSizeY is equal to 64

| Level: |  |  |  | 6.3 |
| --- | --- | --- | --- | --- |
| Max luma picture size (samples): |  |  |  | 80 216 064 |
| Max luma sample rate (samples/sec) |  |  |  | 4 812 963 840 |
| Format nickname | Luma width | Luma height | Luma picture size |  |
| 1080 HD | 1 920 | 1 080 | 2 088 960 | 960.0 |
| 2K×1K | 2 048 | 1 024 | 2 097 152 | 960.0 |
| 2K×1080 | 2 048 | 1 080 | 2 228 224 | 960.0 |
| 4XGA | 2 048 | 1 536 | 3 145 728 | 960.0 |
| 16VGA | 2 560 | 1 920 | 4 915 200 | 960.0 |
| 3616×1536 (2.35:1) | 3 616 | 1 536 | 5 603 328 | 858.9 |
| 3672×1536 (2.39:1) | 3 680 | 1 536 | 5 701 632 | 844.1 |
| 3840×2160 (4*HD) | 3 840 | 2 160 | 8 355 840 | 576.0 |
| 4K×2K | 4 096 | 2 048 | 8 388 608 | 573.5 |
| 4096×2160 | 4 096 | 2 160 | 8 912 896 | 540.0 |
| 4096×2304 (16:9) | 4 096 | 2 304 | 9 437 184 | 510.0 |
| 4096×3072 | 4 096 | 3 072 | 12 582 912 | 382.5 |
| 7680×4320 | 7 680 | 4 320 | 33 423 360 | 144.0 |
| 8192×4096 | 8 192 | 4 096 | 33 554 432 | 143.4 |
| 8192×4320 | 8 192 | 4 320 | 35 651 584 | 135.0 |
| 11520×6480 | 11 520 | 6 480 | 74 649 600 | 64.0 |
| 12288×6144 | 12 288 | 6 144 | 75 497 472 | 63.7 |
| 12288×6480 | 12 288 | 6 480 | 79 626 240 | 60.0 |